MMX-instruction set summary.

By Alexander Hugestrand.

Combine the instruction prefix with the options and the suffix to get the desired instruction. In the options and the suffixes, L stands for ”Low” or ”Low order”, and H for ”High” or ”High order”.

Suffix description:		Options description:

1.	B = Byte (8 bit)	1.	- means ”no options”. (”Wrap around mode” in PADD and PSUB).

2.	W = Word (16 bit)	2.	S = ”Signed with saturation”. (Only with PADD and PSUB).

3.	D = Double Word (32 bit)	3.	SS = ”Signed with saturation”.

4.	Q = Quad Word (64 bit)	4.	US = ”Unsigned with saturation”.

5.	- means ”no suffix”.	5.	EQ = Equals.

6.	GT = Greater than.

| Instruction prefix | +options | +suffix | Execution cycles | Description |
| --- | --- | --- | --- | --- |
| PADD PSUB | - / S / US | B / W / D | 1 | Packed eight bytes (B), four words (W), or two double words (D) are added or subtracted in parallel. |
| PMUL | - | LW / HW LUW* / HUW* | Latency: 3 Throughput: 1 | Packed four signed or unsigned 16-bit words are multiplied in parallel. The low-order or high-order bits of the 32-bit result chosen as the result. |
| PMADDWD | - | - | Latency: 3 Throughput: 1 | Packed Multiply Add. Calculates the products of the signed words of the source and the destination operands. The four intermediate 32-bit doubleword products are summed in pairs to produce two doubleword results. |
| PCMP | EQ / GT | B / W / D | 1 | Packed eight bytes, four words, or two d-word elements are compared in parallel. The result is a bitmask with ones (for ”true”) or zeroes (for ”false”) stored in the destination operand. Logical operations can use the mask to select elements. Can be used to implement a packed conditional move operation without a branch. |
| PUNPCK | L / H | BW / WD / DQ | 1 | Unpack Low/High Packed Data. Converts bytes to words, words to doublewords, or doublewords to quadwords. |
| PACK | SS / US | WB / DW | 1 | Converts words into bytes or doublewords into words. |
| PAND PANDN POR PXOR | - | - | 1 | Bitwise logical operations on 64-bit quantities. |
| PSLL PSRL | - | W / D / Q | 1 | Shift left / right logical. Fills the empty high or low order bit with zeros. |
| PSRA | - | W / D | 1 | Shift right arithmetic. Copies the sign bit into empty bit position on the upper end of the operand. |
| MOV | - | D / Q | 1 (if data in cache) | Moves 32 or 64 bits to and from memory to MMX registers, or between MMX registers. 32 bits can be moved between MMX- and integer registers. |
| EMMS | - | - | Varies by implementation. | Empty FP register tag bits. This instruction must be used at the end of an MMX routine before calling other routines that can execute floating point instructions. |

* Not MMX-standard. The instruction is really part of the SIMD instruction set (Pentium III or later).